CSS 422

Jun Hyung Park

Homework 3

Q1.(5 pts) Consider the logic gate circuit shown below.

(1) Derive a Boolean equation for the output X. You don't need to simplify this equation, but feel free to try!

(2) Draw a truth table for the circuit.

(3) Make the same circuit with logisim . Submit the image (export image) and circuit file.

File had been attached in Zip folder

(4) Generate the truth table with logisim(In logisim, project-->analyze circuit-->table). Copy & past the table in your submission.

Q2. (5 pts) Design a combinational circuit system.

(1)  x,y,z are inputs and A,B,C are outputs.Draw a truth table for the given function.

(2) Based on the truth table you draw, build Karnaugh maps for the output A,B,C.

Output For A

Y + XZ

Output For B

~X~Y + ~Y~Z + XYZ

Output For C

~XZ + X~Z

(3) Derive (as simple as possible) Boolean equations for A,B,C using the Karnaugh maps

A) Y + XZ

B) ~X~Y + ~Y~Z + XYZ

C) ~XZ + X~Z

(4) Based on the Boolean equations, draw the logical gate diagram (circuit) for this system in Logisim. Submit the image (export image) and circuit file.

Circuit file will be submitted in zip folder.

(5) Test your circuit with the Logisim simulation and generate the truth table (In logisim, project-->analyze circuit-->table).  Copy & past the table, and compare it with your own table from sub-question (2)

I did compare, it looks same.

Q3. (5 pts)  Combinational circuit and Full Adder.

A full-adder is a combinational circuit that forms the arithmetic sum of three input bits. It consists of three inputs, x, y, z, and two outputs, C and S. Two of the input, that is, x and y, represent the two significant bits to be added. The third input, z, represents the carry from the previous lower significant position. The output S denotes the sum of two bits and C denotes carry. Answer the following sub-questions.

(1 ) Construct a truth table for Full-Adder

(2)  Based on a truth table, construct a K-map for the output S and derive a Boolean equation using K-map. Make the equation as simple as possible

(3)  Based on a truth table, construct a K-map for the output C and derive a Boolean equation using K-map. Make the equation as simple as possible

(4) By algebraic manipulation, show that S can be expressed as the exclusive-OR of the three input variables. That is, show that,

S = (x XOR  y)  XOR   z.

You can prove by deriving from S = (x XOR  y)  XOR   z to the answer you got in the question (2)

(5) By algebraic manipulation, show that C can be expressed as the following term.

C = xy + (x XOR  y)z.

You can prove by deriving from C = xy + (x XOR  y)z.   to the answer you got in the question (3)

(6) Based on the (4) (5), draw a circuit for the full-adder in Logisim simulator and submit the image (export the image) and circuit file.

Circuit file will be attached in folder

Q4. (5 pts) Design a sequential circuit.

A sequential circuit has one D flip-flop and one JK-flip-flop, two inputs x and y, and one output z. A and B are the outputs of each D flip-flop, and JK-flip-flop, respectively. The flip-flop input equations and the circuit output are as follows. Here DA is the D input of the D-flip flop of A, and JB, KB is the J and K input of the JK-flip flop of B.

DA = ~xy + yB

JB = ~xB + xy

KB = yB + ~xA

z = y+x~y

(1) Draw the logic diagram of the circuit and test it with Logisim. Please attach the circuit image, circuit file, and attach the generated table.

The circuit file will be attached.

Table it generates in Logisim

State Table I worked on by Testing

(2) Construct a state diagram of this circuit.

Q5. (5 pts) Design a system

Design a system with the following state changes: This is a sequential circuit with three flip-flops. The state sequence is changed with a clock as in the order of, 111,110,101,100,011,010,001,000,111 and repeat. Use JK flip-flops.

(1) Draw a state diagram

(2) Construct an excitation table

(3) Draw K-maps and derive Boolean equations using K-maps. Make the equations as simple as possible

(4) Draw the system in Logisim simulator and include the circuit image and circuit file .

Circuit file will be provided.

(5) Test the system and include the generated table .

The output of the table matches with FSM, therefore it is correct.

Extra points (5 pts).  Sequencial circuit and a full-adder

The following sequential circuit includes a full-adder (described in the previous question). Inputs are X, Y and carry-in,  and outputs are the next state of S and Q.

(1) Implement the sequential circuit in Logisim simulator and submit the  image and circuit file.

(2) Complete the following truth table for the following sequential circuit: Note that the Carry out signal is output, not the input. The carry in signal is the same as the Q. You can change the Carry-in bit by clicking the D-FF.

| X | Y | Carry-in (or Q before clock) | S (before clock) | Carry-out (before clock) | S (after clock) | Carry-out (after clock) |
| --- | --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 0 | 0 | 1 | 1 | 1 | 1 | 1 |
| 0 | 1 | 0 | 1 | 1 | 0 | 0 |
| 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 1 | 0 | 0 |
| 1 | 0 | 1 | 0 | 0 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0 | 0 |
| 1 | 1 | 1 | 1 | 0 | 0 | 0 |